# PawPortion Hazard Considerations

This document describes hazards associated with preparing a pet meal using the PawPortion Android app and scale. Each hazard identifies a possible failure and consequence, with references to the relevant user requirements and acceptance criteria in PP-URS-001, revision 1.0, 20 October 2026.

# Hazard considerations

| ID | Item | Hazard consideration |
| --- | --- | --- |
| H-01 | Bluetooth loss or stale data | The owner trusts an old weight during a connection interruption and prepares an incorrect portion. |
| UR-009 UR-013 | Existing coverage | Disconnected, invalid or unavailable readings are not presented as live. Finish stays unavailable until a fresh valid stable weight is available after reconnect. AC-009.3–AC-009.5; AC-013.3–AC-013.5. |

| ID | Item | Hazard consideration |
| --- | --- | --- |
| H-02 | Invalid or unstable measurement | A faulty or moving measurement is treated as successful tare or final weight, resulting in an incorrect portion. |
| UR-008 UR-011 UR-015 | Existing coverage | Stable tare is confirmed before Add Food. Final capture requires a valid stable weight. Problems are distinguished from portion status and do not appear as success. AC-008.1–AC-008.4; AC-011.1; AC-015.1–AC-015.2. |

| ID | Item | Hazard consideration |
| --- | --- | --- |
| H-03 | Incorrect bowl reference | An old or incorrect bowl reference makes the displayed food weight include or omit part of the load. |
| UR-007 UR-009 UR-017 | Existing coverage | Setup captures an unloaded reference. Every meal requires fresh tare. Displayed food weight excludes that meal’s bowl reference. AC-007.1–AC-007.2; AC-009.1; AC-017.1–AC-017.2. |

## Hazard considerations continued

| ID | Item | Hazard consideration |
| --- | --- | --- |
| H-04 | Wrong or unconfirmed settings | A per-meal target is misunderstood, or an unsuccessful save is treated as accepted, leading to an unintended portion. |
| UR-004 UR-005 | Existing coverage | The owner supplies the per-meal target with the specified guidance. Start Meal waits for confirmed saved target and tolerance; failed saves retain entries and offer Retry. Accepted settings persist. AC-004.4; AC-004.6–AC-004.9; AC-005.1–AC-005.3. |

| ID | Item | Hazard consideration |
| --- | --- | --- |
| H-05 | Wrong scale selected | The owner selects another nearby scale and relies on measurements from the wrong weighing surface. |
| UR-001 UR-002 UR-003 | Existing coverage | Setup identifies devices by name or suffix and allows selection. The selected scale is remembered; the owner can explicitly forget or change it. AC-001.1–AC-001.3; AC-002.2; AC-003.2–AC-003.3. |

| ID | Item | Hazard consideration |
| --- | --- | --- |
| H-06 | Misleading portion indication | Units, sign, rounding or range feedback mislead the owner into preparing or accepting an unintended portion. |
| UR-010 UR-011 UR-016 | Existing coverage | Weights use labeled grams and two decimals; sign and comparisons are preserved. Text accompanies color. Outside-range completion requires a warning and Save Anyway. AC-010.1–AC-010.4; AC-011.3–AC-011.5; AC-016.5–AC-016.10. |

| ID | Item | Hazard consideration |
| --- | --- | --- |
| H-07 | Bowl removal during a meal | Continuing after the bowl is removed or replaced can make the existing tare inappropriate for the portion. |
| UR-019 | Existing coverage | Bowl Placed arms detection. Valid readings within ±0.5 g of the unloaded reference continuously for more than three seconds trigger removal; preparation restarts with a fresh meal and tare. AC-019.1–AC-019.3. |