# PawPortion Technical Specification

| Control | Value |
| --- | --- |
| Document ID | PP-TS-001 |
| Revision and date | 1.0 ¦ 21 October 2026 |

PawPortion combines an electronic scale with an Android app to help an owner prepare pet food to a chosen weight. This specification identifies the hardware, firmware, software and operating constraints needed for that product.

# 1. Product boundary

The owner supplies the food target and adds food manually. The scale measures weight and applies meal rules. Android provides setup, meal guidance, Settings and a History tab. Weight entry and display use grams with two decimal places.

The product includes temporary BLE reconnection while the app remains open and the scale remains powered. iOS, cloud services, automatic dispensing, feeding recommendations, app-closure recovery and recovery from scale power loss during a meal are outside this scope.

# 2. Hardware

| Item | Selection and purpose |
| --- | --- |
| Nordic controller | nRF52840 for local processing, Bluetooth Low Energy communication and storage of retained scale settings. |
| Development board | nRF52840 DK for building, programming and evaluating the firmware. |
| Measurement module | M5Stack Mini Scales module with onboard STM32 and HX711 measurement electronics, connected to the Nordic controller over I2C. Confirm the exact model and revision. |
| Weighing surface | A surface that supports the empty bowl and food together. Bowl weight and any supported platform count toward total physical load. |
| Power and connections | A power supply and I2C electrical arrangement compatible with both the controller and measurement module. Supply details and any level translation must be selected before powered integration. |
| Owner device | An Android phone with Bluetooth Low Energy support. The minimum Android version remains to be selected. |

# 3. Firmware and software

| Element | Technology and technical need |
| --- | --- |
| Scale firmware | C using Nordic nRF Connect SDK and Zephyr, running on the nRF52840. |
| Sensor access | Use the Mini Scales I2C interface to obtain readings. Keep sensor access separate from meal calculations and Bluetooth communication. |
| Android application | A native Android app for device setup, meal preparation, settings and history. Language, UI toolkit and minimum operating-system version remain to be selected. |
| Device communication | Bluetooth Low Energy with a custom GATT application interface for measurements, settings and meal interactions. Detailed message formats belong in the protocol specification. |
| Phone persistence | A local Android database for completed meals. The database library remains to be selected. No cloud service is required. |
| Development support | Nordic build and programming tools, a firmware debug/log connection, and Android build and device-testing tools. Tool versions and commands belong in development setup instructions. |

# 4. Information and storage

| Information | Technical need |
| --- | --- |
| Target and tolerance | Retain accepted values on the scale across meals, disconnections and power cycles. Confirm saving before Android treats them as accepted. |
| Weight references | Keep the unloaded scale reference distinct from each meal’s empty-bowl tare. Each meal requires a fresh tare. |
| Active meal | Keep the active meal and tare on the scale while it remains powered, including during a temporary BLE interruption. A restart requires a new meal and tare. |
| Completed meals | Store history in Android’s local database, including finish time, final net weight, target, tolerance and portion status. The scale does not maintain a meal-history log. |
| Remembered scale | Retain the information needed for Android to reconnect to the selected scale. Keep an optional friendly name in the app. |

# 5. Operating constraints

Measurement data must distinguish a usable reading from a missing, stale or faulty reading. Preserve signed weight values. Two decimal places describe representation and display; they do not establish weighing accuracy. Capacity, accuracy and stable-measurement criteria require their own numerical limits and physical evaluation.

The total load includes the bowl and food. Tare removes the bowl’s contribution from the displayed food weight; it does not increase the hardware’s physical load capacity. An unloaded reference and an empty-bowl tare serve different purposes and must remain separate.

Device setup must support pairing and remembering the scale without PIN entry. Later use reconnects to that scale without repeating device selection. The detailed security configuration, stable identity and bond-management behavior require selection before that connection experience is built.

Connection loss must be visible to the owner. Cached or invalid data must not appear as live weight. After reconnecting, the app obtains current information before allowing actions that require a fresh, valid, stable reading.

# 6. Technical selections still needed

| Selection | What must be established |
| --- | --- |
| Sensor and load limits | Exact Mini Scales model and revision, usable total load and operating margin. |
| Power arrangement | Supply source, compatible voltage levels, connectors and any I2C level translation. |
| Measurement performance | Required accuracy, calibration approach and criteria for accepting a stable measurement. |
| Android foundation | Minimum Android version, application language, UI toolkit and local database technology. |
| Connection identity and security | Device identity, bonding and encryption policy, and how a remembered scale is forgotten or replaced. |

These selections do not require a complete internal design. Detailed algorithms, data schemas, task scheduling and recovery mechanisms are defined when the corresponding feature is designed.